Design of Cyclic Code Encoder and Decoder Using Polynomial Arithmetic in MATLAB

1. Aim

To design a Cyclic Code encoder and decoder using polynomial arithmetic in MATLAB and verify the generated codeword.

2. Theory

A cyclic code is a linear block code in which any cyclic shift of a valid codeword is also a valid codeword.

Let:

- Message polynomial = mx

- Generator polynomial = gx

- Codeword polynomial = cx

- Code length = n

- Message length = k

- Number of parity bits = r=n-k

The codeword is generated using:

cx=mxxr+px

where pxis the remainder obtained by dividing mxxrby gx.

Thus,

px=remmxxrgx

and

cx=mxxr+px

All polynomial operations are performed using modulo-2 arithmetic, where:

1+1=0

3. Encoder

Encoding steps

- Select the message bits.

- Select a generator polynomial gx.

- Calculate r=n-k.

- Multiply the message polynomial by xr.

- Divide the resulting polynomial by gx.

- Find the remainder px.

- Add the remainder to obtain the codeword.

Example

Consider:

m=1 0 1 1

and generator:

gx=x3+x+1

Therefore:

g=1 0 1 1

Here,

k=4, r=3, n=7

The message is multiplied by x3:

mxx3

Then divide by gxusing modulo-2 polynomial division.

The resulting codeword is:

c=1 0 1 1 0 0 1

4. MATLAB Program:

Output:

Cyclic Code Decoder

At the receiver, the received codeword is divided by the same generator polynomial.

Let the received polynomial be:

rx

Calculate:

Sx=rx  mod  gx

where Sxis called the syndrome.

Decision

- If syndrome = 000, there is no detectable error.

- If syndrome ≠ 000, an error is detected.

For the transmitted codeword:

1011001

division by gxgives:

Sx=000

Therefore, the received codeword is valid.

6. MATLAB Decoder Program

Output:

7. Verification of Codeword

For the selected message:

m=1011

Generator:

gx=x3+x+1

The encoder produces:

c=1011001

Now,

1011001÷1011

gives:

Remainder=000

Therefore, the generated codeword satisfies the cyclic-code condition.

cx  mod  gx=0

Hence, the codeword generation and decoding are successfully verifie

9. Result

Thus, a Cyclic Code encoder and decoder were successfully designed using polynomial arithmetic in MATLAB. For the message 1011 and generator polynomial gx=x3+x+1, the generated codeword is 1011001. The zero syndrome verifies that the codeword is valid.